| EX NO: 4 | Configure SMTP and FTP protocols using Packet Tracer |
| --- | --- |
| DATE:04/08/26 |  |

# Objectives

- To configure and test email communication between networked devices using the SMTP protocol and verify successful message delivery through a mail server.

- To configure and test file transfer operations between a client and a server using the FTP protocol and verify successful file upload and download.

# Software/Apparatus required:

  - Cisco Packet Tracer

  - End Devices (PCs)

  - Switches

  - Copper Straight-through Cable / Automatic Cable Selector

  - Routers

  - Server (Email server or FTP Server)

# Procedure

Objective 1: SMTP Protocol

    - Create the network topology using two PCs, switches, a router, and a server in Cisco Packet Tracer.

    - Configure the necessary network settings on all devices and ensure that the router interfaces are enabled.

    - Verify connectivity between the PCs and the server using the ping command.

    - Open the server configuration window and enable the SMTP and POP3 services.

    - Create email accounts for the users on the server by specifying usernames and passwords.

    - Open the Email application on each PC and configure the email account details, including the email address, username, password, and mail server information.

    - From the sender PC, compose a new email by entering the recipient’s email address, subject, and message content.

    - Click Send to transfer the email to the mail server using the SMTP protocol.

    - On the recipient PC, open the Email application and click Receive to retrieve the email from the server using the POP3 protocol.

    - Verify that the email is successfully received in the recipient’s inbox.

    - Repeat the process with different users, if required, to confirm successful email communication.

# Objective 2: FTP Protocol

- Create the network topology using PCs, switches, a router, and an FTP server.

- Configure the network settings and verify connectivity between the client PC and the FTP server.

- Enable the FTP service on the server and create the required files.

- Open the Command Prompt on the client PC and connect to the FTP server using ftp

<Server_IP_Address>.

- Display the files available on the server using dir.

- Download a file from the server using get sample.txt.

- Verify the downloaded file using dir.

- Upload a file from the client PC to the server using put report.txt.

- Display the updated file list using dir.

- Close the FTP session using quit.

- Verify that the file transfer operation was completed successfully.

Network Diagram Objective 1: SMTP Protocol

Objective 2: FTP Protocol

# Output

Objective 1: SMTP protocol

| From address | To address | Message |
| --- | --- | --- |

Composed Email output

Received Email output

Objective 2: FTP protocol

| Username 1 | Password 1 | Username 2 | Password 2 | File shared |
| --- | --- | --- | --- | --- |

# Result

Objective 1: SMTP Protocol

- Email communication was successfully established, and email messages were transmitted and received correctly through the mail server using SMTP.

# Objective 2: FTP Protocol

- File transfer was successfully established, and files were uploaded to and downloaded from the FTP server using the FTP protocol.

# Test case Question 1

Implement the specified SMTP topology using Cisco Packet Tracer and analyze email communication between client devices through a mail server.

Case 1

  - Number of PCs: 2

  - Number of Switches: 1

  - Number of Routers: 1

  - Number of Mail Servers: 1

  - Services Enabled: SMTP and POP3 Case 2

  - Number of PCs: 4

  - Number of Switches: 2

  - Number of Routers: 1

  - Number of Mail Servers: 1

  - Services Enabled: SMTP and POP3

  - Inter-switch Connection: Switch-to-Switch Link Tasks

- Construct the specified topology and configure the mail server with SMTP and POP3 services.

- Create email accounts for all users and configure the email clients on the PCs.

- Send and receive email messages between  users and verify  successful  email communication.

| Aim (10) | Networking Diagram (30) | Procedure (10) | Results (20) | Record (10) | Viva (20) | Total (100) |
| --- | --- | --- | --- | --- | --- | --- |

# Test case Question 2

Implement the specified FTP topology using Cisco Packet Tracer and analyze file transfer operations between client devices and an FTP server.

Case 1

  - Number of PCs: 2

  - Number of Switches: 1

  - Number of Routers: 1

  - Number of FTP Servers: 1

  - File Transfer Mode: Client–Server Case 2

  - Number of PCs: 4

  - Number of Switches: 2

  - Number of Routers: 1

  - Number of FTP Servers: 1

  - File Transfer Mode: Client–Server

  - Inter-switch Connection: Switch-to-Switch Link Tasks

- Construct the specified topology and configure the FTP server with user access and files.

- Establish FTP connectivity between the client PCs and the server.

- Perform file upload and download operations and verify successful file transfer.

| Aim (10) | Networking Diagram (30) | Procedure (10) | Results (20) | Record (10) | Viva (20) | Total (100) |
| --- | --- | --- | --- | --- | --- | --- |